- IP camera components properties and interface

Version:2.22

- Properties List

- ShowTitle

Show the title of the video window or not

- ShowStatus

Show the status of the video window or not

- ShowToolBar

Show the toolbar of the video window or not

- ShowOSDName

Show the device name in the video display area or not

- ShowOSDTime

Show current time in the video display area or not

- ShowFPS

Show frame rate on the status bar of the video window or not

- ShowRate

Show the internet speed on the status bar of the video window or not

- Lan

Language used in ActiveX file

Current support languages

Simple Chinese

English

Traditional Chinese

- Ver

Current Version of the ActiveX file

Read only Property

- DevState

The status of current ipcam

| Returned value | Presenting status |
| --- | --- |
| -1 | No camera added |
| 0 | No work |
| 2 | Camera disconnected |
| 3 | Camera connected |

Read only Property

- IsRecing

Record properly or not

Read only Property

- TurnImg

Video can be turned over or not

- CanPopupMenu

Menu can appear by clicking right button on the video window or not

- SavePath

Video record and capture save path,，when transferring StartRec or Capture, if appointing the saving file parameter as savepath，then video or picture will be saved under the SavePath category，and file name will use the current time automatically。The default category is My Documents，If no setting before，a dialogue box will be appeared for your selecting savepath.

- Selected

Selected or not，when multi-demo exist, then use this property to choose a certain one, so that control it

- EnableAlarm

Enable alarm notify. If alarm is enable, the event OnAlarm will be triggered on alarm. Note: this function is valid on using device code to access device.

- RecMpeg4

Record with MPEG4 format. It is valid for MJPEG IPCam.

- IsFullScreen

Whether in full screen.

- ConnMode

The mode of connecting to ip camera.

| ConnMode | Connect mode |
| --- | --- |
| 0 | Directly |
| 1 | Transmitted by Server |
| 2 | P2P mode |

Note: this property is valid for M serials now

- TransServer

Transmitting server IP address and prot. It is valid on ConnMode with 1.

- VideoFixedRate

Show video with video resolution rate.

- IsTalking

The status of talking.

- Listen

The status of listen.

- VideoRecv

Whether receiving video and audio data.

If set it to false, the activeX also connect the ip camera and receive the alarm notification.

This property is valiable for F series and the TCP mode must be set 1.

- PTZProtocol

The PTZ decoder protocol. 0 is for no PTZ. 1 is for Pelco-D, 2 is for Pelco-P。

*it is valid after AddDev.

- VideoRate

Set the max video rate.

*it is valid after AddDev and before ConnectAll.

- Method List

- AddDev4

Add IP camera

Parameters Instruction:

| Parameter | Type | Instructions |
| --- | --- | --- |
| nDevType | whole | Device type, please refer to the form below |
| bLan | whole | Connect in LAN or not，not 0 means LAN mode. |
| szID | alphabetic string | Device code of the camera，if visiting by LAN mode, then LAN IP address will be filled in.。 |
| nTCPPort | Whole | Reverse，set to 0 |
| szDevName | Alphabetic string | Name of the camera，displayed on the up left side of the video window |
| szAcc | Alphabetic string | Authorized account |
| szPwd | Alphabetic string | Authorized password |
| nChannelID | whole | Path number，first path is 1, and so forth。 |

Returned value is 0；

Types of IP cameras

| Types of IP cameras | Definetion |
| --- | --- |
| Without appointing | 0 |
| E series | 901 |
| F series | 902 |
| M series | 903 |
| V series | 904 |
| F1 series | 906 |
| H3 series | 912 |
| H series | 918 |

- ConnectAll

Connect the IP cameras

No parameter

No returned value

- DisConnectAll

Disconnect the IP cameras

No parameter

No returned value

- DevSet

Display IP camera setting page

No parameter

Returned value

| returned value | Status |
| --- | --- |
| 0 | Normal display, click confirm to return |
| -1 | Normal display, click cancel to return |
| -2 | Cannot display the window |

- NetSet

Display network setting page

No parameter

returned value

| returned value | Status |
| --- | --- |
| 0 | Normal display, click confirm to return |
| -1 | Normal display, click cancel to return |
| -2 | Cannot display the window |

- ClearDevs

Clean IP camera settings

No parameters

No returned value

- StartRec

Start recording。

Parameter Instruction:

| Parameter | Type | Instruction |
| --- | --- | --- |
| szFileName | Alphabetic string | Path of saving record files, if szFileNam is empty，an option dialogue box will be appeared for selecting saving path。If szFileName is savepath，files will be saved in SavePath |

Returned value

| Returned value | Status |
| --- | --- |
| -1 | No IP camera added or connected |
| -2 | Video save path is empty |
| 0 | normal |

- StopRec

Stop recording

No parameter

Returned value:

| Returned value | Status |
| --- | --- |
| -1 | No IP camera added or connected |
| 0 | Normal |

- Capture

Capture current pictures。

Parameter instruction:

| Parameter | Type | Instruction |
| --- | --- | --- |
| szFileName | Alphabetic string | Picture saving path，can save picture with bmp or jpg format，judge it according to the extension name。If szFileName is empty，an option dialogue box will be appeared for selecting saving path。If szFileName is savepath，files will be saved in SavePath |

Returned value

| Returned value | Status |
| --- | --- |
| -1 | No IP camera added or connected |
| -2 | No picture captured。 |
| -3 | Saving file error, maybe the disk is unable to write or is full |
| 0 | Normal |

- CaptureTool

Display the capture tool of the plug in。

No parameter

Returned value:

| Returned value | Status |
| --- | --- |
| -1 | No IP camera added or connected |
| 0 | Normal |

- FullScreen

Display the video window in full screen

No parameter

No returned value

- PTZ

Pan/Title control

Parameter:

| Parameter | Type | Instruction |
| --- | --- | --- |
| nPro | int | PTZ Protocol 0:Pelco-D，1:Pelco-P，Default is Pelco-D |
| nAddr | int | Address of decoder board, default is 1 |
| nDir | int | Actions, see following table |
| nSpeed | int | Speed,1~255 |
| nParam | int | Parameter for preset |
| bWithStop | int | Whether send stop action after action 0:no, 1:yes （Just M series support this parameter） |

No returned value

PTZ actions:

| - nDir value | - action |
| --- | --- |
| - -1 | - Stop |
| - 0 | - Up |
| - 1 | - Left |
| - 2 | - Right |
| - 3 | - Down |
| - 4 | - Focus+ |
| - 5 | - Focus- |
| - 6 | - Zoom- |
| - 7 | - Zoom+ |
| - 8 | - Autoscan on |
| - 9 | - Autoscan off |
| - 10 | - Aperture- |
| - 11 | - Aperture+ |
| - 12 | - Light off |
| - 13 | - Light on |
| - 14 | - Goto preset position nParam |
| - 15 | - Set preset position n |
| - 16 | - Delete preset position n |
| - 17 | - Up-left |
| - 18 | - Down-right |
| - 19 | - Up-right |
| - 20 | - Down-left |

- StartTalk

Start talking

No parameter

No returned value

- StopTalk

Stop talking.

No parameter

No returned value

- Events:

- OnAlarm

Called on IP camera alarming.

Parameter:

| Parameter | Type | Instruction |
| --- | --- | --- |
| bStart | Integer | 1表示开始报警，0表示报警结束。报警时间长度由配置文件决定。 |

Note: It is valid when the F serials is connected with IP address and SN, the E,M serials is connected with SN.

- OnVideoConnected

Called when the IP camera is connected.

Parameter:

| Parameter | Type | Instruction |
| --- | --- | --- |
| BmpInfo | Integer | BITMAPINFOHEADER structure pointer |
| AudioFmt | Integer | AUDIOFOMRATEX structure pointer |

- OnVideoDisconnect

Called when the IP camera is disconnected.

No parameters.

- OnVideoSample

Called on receiving a video or audio frame data.

Parameter:

| Parameter | Type | Instruction |
| --- | --- | --- |
| nFrameID | Integer | Video or Audio frame index |
| IsVideo | Integer | 1:video, otherwise audio |
| pData | Pointer | Video or audio data pointer, PBYTE |
| nDataLen | Integer | Video or audio data length |

- Invalid interface（these interface is invalid）

- Start

- SwitchLayout

- Real file list

| - Files | - Instruction |
| --- | --- |
| ipcamax.ocx | ActiveX file, need to register when instalation |
| AudioCodec.dll | Audio process |
| AviWriter.dll | Record process |
| Com1024.dll | Related to IP camera communication |
| ddisp.dll | Related to display |
| ipcamax.lb5 | Traditional Chinese |
| ipcamax.lcn | Simple Chinese |
| ipcamax.len | English |
| ijl15.dll | JPEG function base |
| LPng.dll | Image function base |
| oemopt.dat | Configuration file |
| Option.ini | Configuration file |
| vcore.dll | Related to video |
| vvfw.dll | Related to video |
| ZPlayer.exe | Video player, program transferring method ZPlayer.exe file play path |
| ZPlayer.lb5, ZPlayer.lcn, ZPlayer.len | Video play language file |

These files should be in the same category when assign them.